# AXI Streaming Bridge -RX Functional Design Specifications

Design and Implementation of CSI Controller IP

Version 4.0.1

Block Owner

Hana Samy Ahmed

Authors

Hana Samy Ahmed

This Page is left Blank Intentionally

# Table of Contents

1	Table of Contents	1

2	Revision History	2

3	Overview	3

4	Operation and Description	3

4.1	Digital Interface	3

4.1.1	Parameters Names	3

4.1.2	Ports Names	3

4.2	Functional Description	3

4.3	Block Diagram	3

4.4	Timing Diagram	3

4.5	Verification Requirements	3

# Revision History

| Version | Date | Author(s) | Revision Notes | Owner Approval |
| --- | --- | --- | --- | --- |
| 1 | 9-2-2026 | Hana Samy |  |  |

# Overview

The AXI Streaming Bridge acts as an interface adapter between the custom internal video pipeline (coming from the Asynchronous FIFO) and the standard system interconnect (AXI4-Stream).

Its primary function is width adaptation and protocol compliance. It takes 24-bit pixel data (RGB888), pads it to 32 bits (standard word alignment), and manages the AXI VALID/READY handshake to ensure seamless data transfer to the downstream system (e.g., DMA or Frame Buffer).

# Operation and Description

## Digital Interface

### Parameters Names

| Parameter Name | Default | Description |
| --- | --- | --- |
| P_PIX_WIDTH | 24 | Must match FIFO Output Width. |
| P_AXI_WIDTH | 32 | Width of the Master AXI Bus. |

### Ports Names

| Port Name | Port Width | Port Type | Description |
| --- | --- | --- | --- |
| System |  |  |  |
| i_aclk | 1 | In | AXI System Clock. |
| i_aresetn | 1 | In | AXI System Reset. |
| Input (From FIFO) |  |  |  |
| i_pix_data | 24 | In | Pixel Data. |
| i_pix_last | 1 | In | End of Line Flag. |
| i_pix_user | 1 | In | Start of Frame Flag. |
| i_pix_valid | 1 | In | Pixel Valid. |
| o_pix_ready | 1 | Out | Backpressure. Connects to FIFO i_r_ready. |
| AXI Master Output |  |  |  |
| m_axis_tdata | 32 | Out | AXI Data Bus. |
| m_axis_tkeep | 8 | Out | Byte Keep (valid bytes in word). |
| m_axis_tlast | 1 | Out | AXI Packet Boundary (EOL). |
| m_axis_tuser | 1 | Out | AXI User Signal (SOF). |
| m_axis_tvalid | 1 | Out | Master Valid. |
| m_axis_tready | 1 | In | Slave Ready (Backpressure from System). |

## Functional Description

Detailed description about the block functionality should be written here

The block operates as a transparent bridge with combinatorial logic for data formatting and a simple pass-through for flow control.

- Data Width Expansion (Padding):

  - The input i_pix_data is 24 bits wide (3 Bytes).

  - The output m_axis_tdata is 32 bits wide (4 Bytes).

  - Logic: m_axis_tdata[23:0] maps directly to i_pix_data.

  - Padding: m_axis_tdata[31:24] is explicitly driven to 0x00 (Zero Padding).

- Handshake Management (Flow Control):

  - Forward Path: m_axis_tvalid connects directly to i_pix_valid.

  - Backward Path: o_pix_ready connects directly to m_axis_tready.

  - Note: The bridge does not buffer data; if the AXI slave asserts backpressure (m_axis_tready = 0), the bridge immediately asserts backpressure to the FIFO (o_pix_ready = 0).

- Sideband Signals:

  - m_axis_tlast follows i_pix_last (marking the end of a video line).

  - m_axis_tuser follows i_pix_user (marking the start of a video frame).

  - m_axis_tkeep: Driven to a constant value to indicate valid byte lanes.

    - For 24-bit data in a 32-bit container, this is typically set to 0x0F (indicating the whole 32-bit word is a valid transfer, with the upper byte being valid "padding") or 0x07 (indicating only the lower 3 bytes are data).

    - Default Implementation: 8'h0F (Assuming standard 32-bit word alignment requirements).

## Block Diagram

## Timing Diagram

## Verification Requirements

- Data Integrity: Verify that m_axis_tdata[23:0] exactly matches i_pix_data and that m_axis_tdata[31:24] is always 0x00.

- Handshake Compliance:

- Verify o_pix_ready goes low immediately when m_axis_tready goes low.

- Verify m_axis_tvalid does not assert if i_pix_valid is low.

- Boundary Check: Verify m_axis_tlast asserts exactly on the last pixel of the line and m_axis_tuser asserts exactly on the first pixel of the frame.

- Reset Behavior: Ensure o_pix_ready is low (busy) or handles the reset state correctly when i_aresetn is asserted.