# Depacketizer -RX Functional Design Specifications

Design and Implementation of CSI Controller IP

Version 4.0.1

Block Owner

Hana Samy Ahmed

Authors

Hana Samy Ahmed

This Page is left Blank Intentionally

# Table of Contents

1	Table of Contents	1

2	Revision History	2

3	Overview	3

4	Operation and Description	3

4.1	Digital Interface	3

4.1.1	Parameters Names	3

4.1.2	Ports Names	3

4.2	Functional Description	3

4.3	Block Diagram	3

4.4	Timing Diagram	3

4.5	Verification Requirements	3

# Revision History

| Version | Date | Author(s) | Revision Notes | Owner Approval |
| --- | --- | --- | --- | --- |
| 1 | 6-2-2026 | Hana Samy |  |  |

# Overview

The RX Depacketizer serves as the protocol parsing layer of the video receiver pipeline. It receives a continuous raw byte stream, identifies packet boundaries, and splits the stream into its constituent components:

- Header: Extracted and sent to an External ECC Block.

- Payload: Extracted as "Active Video" and sent to the Unpacker.

- Payload + Footer: Sent to an External CRC Block for integrity verification.

The block operates in a "Push-Only" mode (no backpressure on the input) and supports a configurable CRC mode where the interface to the external CRC block can be enabled or disabled to save power.

# Operation and Description

## Digital Interface

### Parameters Names

| Parameter Name | Default | Description |
| --- | --- | --- |

### Ports Names

| Port Name | Port Width | Port Type | Description |
| --- | --- | --- | --- |
| System |  |  |  |
| i_clk | 1 | Input | System Clock. |
| i_rst_n | 1 | Input | Active-Low Asynchronous Reset. |
| Input Stream |  |  |  |
| i_rx_data | 8 | Input | Incoming raw byte stream (Header -> Payload -> Footer). |
| i_rx_valid | 1 | Input | Valid flag for input stream. |
| Output (to unpacker) |  |  |  |
| o_pay_data | 8 | Output | Extracted Active Video payload. |
| o_pay_valid | 1 | Output | Valid flag for video output. |
| o_pay_last | 1 | Output | Asserts on the last byte of the payload (End of Line). |
| o_pay_user | 1 | Output | Asserts on the first byte of a Frame (Start of Frame). |
| External ECC Interface |  |  |  |
| o_ecc_hdr_data | 32 | output | The 4-byte Packet Header sent to external ECC block. |
| External CRC Interface |  |  |  |
| o_crc_data | 8 | Output | Payload byte stream sent to external CRC block (includes Footer).. |
| o_crc_valid | 1 | Output | Valid flag for CRC calculation. |
| o_crc_start | 1 | output | Start marker for external CRC (asserted after header ends). |
| O_crc_end |  |  | End marker for external CRC (asserted on last payload byte). |
| Register Interface |  |  |  |
| i_cfg_crc_en | 1 | Input | CRC Mode: 1=Check CRC, 0=Ignore CRC (Disabled). |
| i_ecc_syndrome | 2 | Input | Response from ECC: 00=OK, 01=1-bit Err, 10=2-bit Err. |
| i_ecc_corr_hdr | 32 | Input | The corrected header (used if 1-bit error detected). |

## Functional Description

The Depacketizer operates as a Finite State Machine (FSM) that coordinates the data flow.

1. Header Phase (ECC Output)

- The block accumulates the first 4 bytes.

- The full 32-bit header is driven on o_ecc_hdr_data to the External ECC block.

2. Payload Phase (Video & CRC Start)

- The FSM streams bytes to both the Unpacker and the External CRC block.

- CRC Control:

  - o_crc_start pulses High on the First Payload Byte.

  - o_crc_valid stays High for the duration of the payload.

  - o_crc_end pulses High on the Last Payload Byte.

- Rationale: The o_crc_end signal tells the External CRC block: "This is the last byte to include in your checksum calculation."

3. Footer Phase (CRC Check)

- Immediately following the payload, the FSM receives the 2-byte Footer.

- To Unpacker: These bytes are dropped (not sent).

- To CRC Block:

  - The Footer bytes are passed to o_crc_data (with o_crc_valid High).

  - o_crc_end remains Low during these two cycles.

- Behavior: This implies the External CRC block uses o_crc_end to latch its calculated value, and then consumes the subsequent 2 bytes on o_crc_data to perform the comparison internally.

## Block Diagram

## Timing Diagram

## Verification Requirements

The verification plan must cover the interaction with the external blocks and the configuration modes:

- Mode Toggling: Verify that o_crc_valid remains Low when i_cfg_crc_en is set to 0, even while valid payload data is flowing to the Unpacker.

- Timing Alignment: Verify o_crc_start aligns exactly with the first payload byte and o_crc_end aligns with the last footer byte.

- Header Extraction: Ensure o_ecc_hdr_data is valid for at least one clock cycle immediately following the reception of the 4th header byte.

- Protocol Separation: Confirm that Header and Footer bytes never appear on the o_pay_data (Unpacker) interface.