# ECC Generator Functional Design

# Specifications

Design and Implementation of CSI Controller IP

Version 4.0.1

Block Owner

Mohamed Zaher Fouda 
Authors

Mohamed Zaher Fouda

This Page is left Blank Intentionally

# Table of Contents

1	Table of Contents	1

2	Revision History	2

3	Overview	3

4	Operation and Description	3

4.1	Digital Interface	3

4.1.1	Parameters Names	3

4.1.2	Ports Names	3

4.2	Functional Description	3

4.3	Block Diagram	3

4.4	Timing Diagram	3

4.5	Verification Requirements	3

# Revision History

| Version | Date | Author(s) | Revision Notes | Owner Approval |
| --- | --- | --- | --- | --- |
| 1 | 20-11-2025 | Mohamed Zaher Fouda |  |  |

3 Overview

The ECC (Error Correction Code) Decoder is a critical data-integrity submodule located within the MIPI CSI-2 RX Controller. It operates on the 32-bit Packet Header (PH) received from the D-PHY (after byte un-packing and synchronization) to ensure the validity of the Low Level Protocol (LLP) routing and length information.

Its primary function is to extract the 24-bit payload (8-bit Data Identifier and 16-bit Word Count) and the 6-bit ECC parity field, then calculate the error syndrome. Using the SEC-DED (Single Error Correction, Double Error Detection) Hamming code matrix, it actively corrects any single-bit flip introduced during transmission, flags uncorrectable multi-bit errors to halt faulty packet processing, and forwards the verified header data to the RX Protocol Engine.

4 Operation and Description

4.1 Digital Interface

4.1.1 Ports Names

| Port Name | Port Width | Port Type | Description |
| --- | --- | --- | --- |
| Clocks & Resets |  |  |  |
| i_byte_clk | 1 | Input | High-speed Byte Clock from RX PHY. |
| i_rst_n | 1 | Input | Active-low Asynchronous Reset. |
| Upstream Interface |  |  |  |
| i_rx_header | 32 | Input | Full 32-bit Packet Header (Byte 0: DI, Bytes 1-2: WC, Byte 3: ECC). |
| i_header_valid | 1 | Input | Valid flag indicating the 32-bit header is available. |
| Downstream Interface |  |  |  |
| o_corrected_di | 8 | Output | Verified/Corrected Data Identifier. |
| o_corrected_wc | 16 | Output | Verified/Corrected Word Count. |
| o_err_status | 2 | Output | Error status: 2'b00 (Clean), 2'b01 (Single Error Corrected), 2'b10 (Fatal Multi-bit Error). |
| o_header_ready | 1 | Output | Valid flag asserting that the corrected data is ready for the Protocol Engine. |

4.2 Functional Description

The architecture of the ECC Decoder relies on a high-speed combinational XOR matrix followed by decoding logic to isolate and resolve errors with minimal latency.

- Syndrome Calculation: * Role: The module intercepts the 32-bit i_rx_header. It applies the MIPI CSI-2 parity matrix across the 24-bit data (DI + WC) and XORs the result with the received 6-bit ECC (i_rx_header[29:24]).

  - Output: A 6-bit syndrome vector. If the syndrome is 6'b000000, the data is perfectly clean.

- Error Detection and Location:

  - Role: Evaluates the non-zero syndrome.

  - Single Error: If the syndrome matches one of the known 30 column vectors of the Hamming matrix (24 data positions + 6 parity positions), a single-bit error is detected.

  - Multi-Error: If the syndrome is non-zero but does not match any valid single-error vector, a multi-bit error is detected.

- Correction and Forwarding:

  - Single Error Correction (SEC): The block generates a 24-bit correction mask with a 1 at the exact index of the faulty bit. This mask is XORed with the raw 24-bit data to flip the faulty bit back to its correct state. o_err_status is set to 2'b01.

  - Double Error Detection (DED): If a multi-bit error is identified, no correction is attempted to prevent data corruption. o_err_status is asserted to 2'b10 to instruct the RX FSM to drop the packet.

  - The final verified DI and WC are pushed to o_corrected_di and o_corrected_wc, and o_header_ready is asserted.

4.3 Block Diagram

Figure 1 ECC Checker Block Diagram

4.4 Timing Diagram

Figure 2 ECC Checker Timing diagram

4.5 Verification Requirements

- Zero-Error Validation: Feed the module mathematically perfect 32-bit headers and verify that o_err_status remains 2'b00 and the output data perfectly matches the input data.

- Exhaustive SEC Testing: Iterate through all 24 data bit positions and all 6 ECC bit positions, intentionally flipping exactly one bit per test. Verify that o_err_status flags 2'b01 and o_corrected_di / o_corrected_wc reflect the original, uncorrupted values.

- DED and Multi-bit Testing: Inject 2-bit, 3-bit, and random multi-bit errors into the 32-bit header. Verify that o_err_status definitively flags 2'b10 (Fatal Error) for all double-bit errors, ensuring the Protocol Engine is warned not to process the corrupted length or routing data.