# MIPI Descrambler Functional Design Specifications

MIPI CSI Controller IP

Version 1.0

Block Owner

Abdelrahman Elsayed

Authors

Abdelrahman Elsayed

This Page is left Blank Intentionally

# Table of Contents

1	Table of Contents	1

2	Revision History	2

3	Overview	3

4	Operation and Description	3

4.1	Digital Interface	3

4.1.1	Parameters Names	3

4.1.2	Ports Names	3

4.2	Functional Description	3

4.3	Block Diagram	3

4.4	Timing Diagram	3

4.5	Verification Requirements	3

# Revision History

| Version | Date | Author(s) | Revision Notes | Owner Approval |
| --- | --- | --- | --- | --- |

# Overview

The Data Descrambler is a critical receive-side module within the MIPI CSI Controller IP. It acts as the direct counterpart to the transmit-side Scrambler. By utilizing an identical 16-bit Linear Feedback Shift Register (LFSR) and symmetric XOR logic, the Descrambler accurately recovers the original plaintext byte stream from the incoming ciphered payload. It operates at a high-speed rate of one byte per clock cycle to maintain pipeline throughput.

# Operation and Description

## Digital Interface

### Parameters Names

| Parameter Name | Default | Description |
| --- | --- | --- |
| LANE_SEED | 16'h0810 | The 16-bit initialization vector (seed) used to synchronize the receive-side LFSR state with the transmit-side. |

### Ports Names

| Port Name | Port Width | Port Type | Description |
| --- | --- | --- | --- |
| i_clk | 1 | Input | System core clock. |
| i_rst_n | 1 | Input | Active-low asynchronous system reset. |
| i_scram_en | 1 | Input | Runtime enable control (1 = descramble data, 0 = passthrough bypass). Must match the TX configuration. |
| i_scrambled_data | 8 | Input | Incoming scrambled (ciphered) byte stream from the RX pipeline. |
| i_lane_vld | 1 | Input | Valid signal indicating active payload data on the input bus. |
| o_descrambled_data | 8 | Output | The recovered plaintext output byte stream. |
| o_lane_vld_out | 1 | Output | The output valid signal, matching the input valid. |

## Functional Description

The MIPI Descrambler relies on the symmetric property of the XOR cipher (where A XOR B XOR B = A). The operation is identical to the Scrambler and features three primary mechanisms:

- Symmetric Keystream Generation (Unrolled LFSR): To keep up with the high-speed data rate, the sequential LFSR equations are hardware-unrolled. Wide XOR logic computes 8 future feedback bits (w_fb[0:7]) concurrently. This allows the LFSR to advance 8 states per clock cycle, generating an 8-bit keystream (w_key_stream) that perfectly mirrors the transmitter's keystream.

- Deterministic Synchronization: For the cipher to be successfully broken, the TX and RX LFSRs must remain perfectly in sync. The internal state (r_lfsr_state) is forced to LANE_SEED during inter-packet idle periods or when the module is bypassed (i_scram_en = 0). During an active transmission, the LFSR only shifts to generate the next key when a valid payload byte is received (i_lane_vld = 1).

- Priority MUX & Cipher Cancellation: The output routing is controlled by a 3-level priority mechanism:

  - If i_lane_vld is 0, the output is forced to 8'h00 to prevent residual data leakage.

  - If i_scram_en is 0, the scrambled data bypasses the XOR logic and flows to the output unaltered.

  - Otherwise, the output is the bitwise XOR operation between the input i_scrambled_data and the w_key_stream. Because the TX applied the exact same keystream, this second XOR operation cancels the cipher and recovers the plaintext.

## Block Diagram

## Timing Diagram

## Verification Requirements