# CSI-2 RX Unpacker Functional Design Specifications

Design and Implementation of CSI Controller IP

Version 4.0.1

Block Owner

Hana Samy Ahmed

Authors

Hana Samy Ahmed

…

This Page is left Blank Intentionally

# Table of Contents

1	Table of Contents	1

2	Revision History	2

3	Overview	3

4	Operation and Description	3

4.1	Digital Interface	3

4.1.1	Parameters Names	3

4.1.2	Ports Names	3

4.2	Functional Description	3

4.3	Block Diagram	3

4.4	Timing Diagram	3

4.5	Verification Requirements	3

# Revision History

| Version | Date | Author(s) | Revision Notes | Owner Approval |
| --- | --- | --- | --- | --- |
| 1 | 31-1-2026 | Hana Samy |  |  |

# Overview

The CSI-2 RX Unpacker is the first processing stage in the CSI-2 Receiver Data Path. It receives a raw, valid byte stream from the CSI-2 Depacketizer (D-PHY interface) and reconstructs the original parallel pixel data.

This block operates entirely on the high-speed i_byte_clk. It supports dynamic "Reverse Gearbox" operations to unpack multi-byte pixel formats (like RGB888 and RAW10) back into a native pixel bus. Its output is designed to be fed directly into an Asynchronous FIFO for clock domain crossing.

# Operation and Description

## Digital Interface

### Parameters Names

| Parameter Name | Default | Description |
| --- | --- | --- |
| MAX_PIXEL_WIDTH | 24 | Maximum width of the reconstructed pixel (e.g., 24 for RGB888, 10 for RAW10). |

### Ports Names

| Port Name | Port Width | Port Type | Description |
| --- | --- | --- | --- |
| Clocks and Resets |  |  |  |
| i_byte_clk | 1 | Input | High-speed MIPI Byte Clock. |
| i_rst_n | 1 | Input | Active-low asynchronous reset. |
| Configuration |  |  |  |
| i_cfg_data_type | 6 | Input | MIPI Data Type (DT) code. Configures the unpacking ratio. |
| Byte Input Interface (From Depacketizer) |  |  |  |
| i_rx_data | 8 | Input | Incoming payload byte stream. |
| i_rx_valid | 1 | Input | Valid flag for input byte. |
| i_rx_packet_done | 1 | Input | Indicates the last byte of the current line (Payload End). |
| i_rx_frame_start | 1 | Input | Single-cycle pulse indicating Start of Frame (from FS Packet). |
| Native Output Interface (To Async FIFO) |  |  |  |
| o_native_data | WIDTH | Output | Reconstructed Parallel Pixel Data. |
| o_native_valid | 1 | Output | Valid flag for the output pixel. |
| o_native_sof | 1 | Output | Start of Frame marker (aligned with the first valid pixel of the frame). |
| o_native_eol | 1 | Output | End of Line marker (aligned with the last valid pixel of the line). |

## Functional Description

The block functions as a configurable deserializer. It implements the inverse logic of the "Pixel-to-Byte Converter" described in the TX specification.

1. Dynamic Unpacking Logic

The state machine behavior changes based on i_cfg_data_type:

- Mode A: Byte Accumulation (RGB888, YUV422)

  - Logic: Accumulates N bytes to form 1 Pixel.

  - Example (RGB888 - 1:3 Ratio):

    - Cycles 1-2: The block receives and stores Byte 0 (Red) and Byte 1 (Green). o_native_valid is LOW.

    - Cycle 3: The block receives Byte 2 (Blue).

    - Output: It concatenates {Byte0, Byte1, Byte2} and asserts o_native_valid for one cycle.

  - Output Rate: 1 Pixel every 3 Clocks (assuming continuous input).

- Mode B: Reverse Gearbox (RAW10)

  - Logic: Unpacks 5 Bytes into 4 Pixels (Reverse of TX Mode B ).

  - Latency Phase (Cycles 1-5):

    - The block receives 4 bytes (MSBs of Pixels 1-4) and finally the 5th byte (Aggregated LSBs).

    - During these 5 input cycles, the block buffers the data.

  - Reconstruction Phase:

    - Once the 5th byte is latched, the block has all the data for 4 pixels.

    - It extracts the LSB pairs (Bits 1:0, 3:2, 5:4, 7:6) from Byte 5 and merges them with the stored MSBs.

  - Burst Output:

    - To match the single-pixel output bus width, the block bursts the 4 pixels out sequentially over the next 4 clock cycles (while the next group of bytes is arriving).

  - Output Rate: 4 Pixels every 5 Clocks.

2. Marker Propagation

- Start of Frame (SOF): The block latches the i_rx_frame_start pulse. When the very first pixel of the frame is fully reconstructed and o_native_valid goes High, o_native_sof is asserted High for that same cycle.

- End of Line (EOL): When i_rx_packet_done is detected on the input (coincident with the last byte), the block tags the last pixel currently being reconstructed. When that last pixel is finally output, o_native_eol is asserted.

## Block Diagram

## Timing Diagram

## Verification Requirements

- Format Reconstruction: Verify that RGB888 inputs (R-G-B sequence) result in a single {R,G,B} output word.

- RAW10 LSB Mapping: Verify that the LSBs from the 5th byte are correctly assigned to Pixels 1, 2, 3, and 4 respectively.

- Throughput Check: Ensure that for RAW10, the output valid ratio is exactly 4/5 (4 valid cycles, 1 gap cycle) when input is continuous.

- Flag Alignment: Verify o_native_sof and o_native_eol assert exactly with the first and last pixels of the line/frame, regardless of the internal unpacking delay.

- Partial Group Handling: (Corner Case) Inject a line that ends with fewer than 4 pixels in RAW10 mode. Verify the block outputs the valid pixels available without hanging or waiting forever for a 5th byte that won't come.