# PPI TX Functional Design Specifications

MIPI CSI Controller IP

Version 1.0

Block Owner

Abdelrahman Elsayed

Authors

Abdelrahman Elsayed

This Page is left Blank Intentionally

# Table of Contents

1	Table of Contents	1

2	Revision History	2

3	Overview	3

4	Operation and Description	3

4.1	Digital Interface	3

4.1.1	Parameters Names	3

4.1.2	Ports Names	3

4.2	Functional Description	3

4.3	Block Diagram	3

4.4	Timing Diagram	3

4.5	Verification Requirements	3

# Revision History

| Version | Date | Author(s) | Revision Notes | Owner Approval |
| --- | --- | --- | --- | --- |

# Overview

The Protocol-to-PHY Interface Transmitter (PPI TX) is a core module within the MIPI CSI Controller IP. It serves as the primary bridge between the internal System Clock domain and the MIPI D-PHY High-Speed (HS) Clock domain. The module manages the transition to HS mode, handles Clock Domain Crossing (CDC) using Asynchronous FIFOs, and implements Ultra-Low Power Management (ULPM) and High-Speed Request sequencing to ensure fully compliant communication with the D-PHY Macro.

# Operation and Description

## Digital Interface

### Parameters Names

| Parameter Name | Default | Description |
| --- | --- | --- |
| N/A | N/A | This module does not utilize top-level instantiation parameters. Internal FIFO dimensions are fixed at WIDTH=8, DEPTH=16. |

### Ports Names

| Port Name | Port Width | Port Type | Description |
| --- | --- | --- | --- |
| i_clk_sys | 1 | Input | System core clock. |
| i_rst_n_sys | 1 | Input | Active-low asynchronous system reset. |
| i_rst_n_tx | 1 | Input | Active-low asynchronous TX/PHY reset. |
| i_active_lanes | 3 | Input | Configuration input defining the active lanes. |
| i_lane[0:3]_data | 8 | Input | Parallel plaintext or scrambled data from the TX pipeline. |
| i_lane[0:3]_vld | 1 | Input | Valid signals indicating active payload data for the corresponding lanes. |
| i_req_hs | 1 | Input | High-Speed mode transition request from the system controller. |
| o_TxClkRequestHS | 1 | Output | PPI High-Speed Clock Request sent to the D-PHY. |
| i_TxClkReadyHS | 1 | Input | PPI High-Speed Clock Ready acknowledgement from the D-PHY. |
| i_TxByteClkHS | 1 | Input | MIPI D-PHY High-Speed Byte Clock (Read domain clock). |
| o_TxRequestHS | 4 | Output | PPI High-Speed Data Request for lanes 0 to 3. |
| i_TxReadyHS | 4 | Input | PPI High-Speed Data Ready acknowledgement for lanes 0 to 3. |
| o_lane[0:3]_data_hs | 8 | Output | High-Speed data buses driving the D-PHY lanes. |
| o_lane[0:3]_valid_hs | 1 | Output | High-Speed valid signals driving the D-PHY lanes. |
| i_Shutdown | 1 | Input | Power-down / Shutdown command from the Register File (APB). |
| i_TxClkEsc | 1 | Input | Escape mode clock from the System PLL. |
| o_TxUlpmClk | 1 | Output | Ultra Low Power Mode (ULPM) Clock status signal to D-PHY. |
| o_TxRequestEsc | 4 | Output | Escape Request signals for lanes 0 to 3. |
| o_TxUlpmEsc | 4 | Output | Ultra Low Power Mode (ULPM) Escape signals for lanes 0 to 3. |

## Functional Description

The PPI TX module operates through several highly integrated sub-blocks to handle clock domains and physical layer protocols:

- High-Speed Clock Request & Keep-Alive Logic: To ensure a clean packet termination at the D-PHY level and prevent jitter, the module maintains the HS clock request (o_TxClkRequestHS) active for a programmable TCLK-POST period (configured here as ~40 cycles) after the core packet request (i_req_hs) drops.

- Data Path & Clock Domain Crossing (CDC): Uses 4 independent Asynchronous FIFOs to safely transport data from the i_clk_sys domain to the i_TxByteClkHS domain. The read operations from these FIFOs are strictly gated by the PPI handshake signals (o_TxRequestHS & i_TxReadyHS), guaranteeing that data is only driven when the PHY is fully ready.

- CIL Power Management Logic (ULPM Control): Dynamically manages the power state of the D-PHY. When the controller is idle (no i_req_hs and the Keep-Alive timer has expired) and i_Shutdown is inactive, the logic automatically triggers Ultra Low Power Mode (ULPM) by driving o_TxUlpmClk, o_TxRequestEsc, and o_TxUlpmEsc.

- Synthesis Optimization & EDA Sinks: Includes internal Dummy Flip-Flops and AND-reduction sinks for i_TxClkEsc and unused flags (like FIFO full states). This satisfies structural gate-level synthesis requirements (e.g., SpyGlass constraints) while ensuring no floating nets exist in the design.

## Block Diagram

## Timing Diagram

Any timing Diagrams that relate inputs/outputs together should be demonstrated here

## Verification Requirements