# Pixel to Byte Converter Functional Design Specifications

Design and Implementation of CSI Controller IP

Version 4.0.1

Block Owner

Hana Samy Ahmed

Authors

Hana Samy Ahmed

This Page is left Blank Intentionally

# Table of Contents

1	Table of Contents	1

2	Revision History	2

3	Overview	3

4	Operation and Description	3

4.1	Digital Interface	3

4.1.1	Parameters Names	3

4.1.2	Ports Names	3

4.2	Functional Description	3

4.3	Block Diagram	3

4.4	Timing Diagram	3

4.5	Verification Requirements	3

# Revision History

| Version | Date | Author(s) | Revision Notes | Owner Approval |
| --- | --- | --- | --- | --- |
| 1 | 16-1-2026 | Hana Samy |  |  |

# Overview

This block serializes parallel pixel data into a sequential byte stream suitable for the MIPI D-PHY controller. It supports dynamic configuration to handle multiple MIPI CSI-2 data types (RGB, YUV, RAW) by adjusting the serialization ratio and flow control at runtime. It also decodes the flow markers to provide precise Packet Control Signals for the downstream Packetizer.

# Operation and Description

## Digital Interface

### Parameters Names

| Parameter Name | Default | Description |
| --- | --- | --- |
| MAX_PIXEL_WIDTH | 24 | Maximum input width (e.g., RGB888 needs 24 bits). |
| OUTPUT_WIDTH | 8 | Output width to the D-PHY (Standard MIPI byte lane). |

### Ports Names

| Port Name | Port Width | Port Type | Description |
| --- | --- | --- | --- |
| i_cfg_data_type | 6 | Input | MIPI Data Type (DT) code. Determines the byte count per pixel. |
| Clocks and Resets |  |  |  |
| i_byte_clk | 1 | Input | The high-speed MIPI Byte clock. All logic in this block runs on this clock. |
| i_rst_n | 1 | Input | Active-low asynchronous reset. |
| Native Input (From AXI Streaming-TX) |  |  |  |
| i_native_data | 24 | Input | Parallel pixel data (RGB). |
| i_native_valid | 1 | Input | Valid indicator for the input pixel. |
| o_native_ready | 1 | Output | Backpressure Signal. Goes LOW while the converter is busy serializing the current pixel . |
| i_native_sof | 1 | Input | Start of Frame marker (aligned with first pixel). |
| i_native_eol | 1 | Input | End of Line marker (aligned with last pixel). |
| Byte Output |  |  |  |
| o_byte_data | 8 | Output | Serialized 8-bit data stream. |
| o_byte_valid | 1 | Output | Valid indicator for the output byte. |
| i_byte_ready | 1 |  | Backpressure from D-PHY |
| o_byte_frame_start | 1 | Output | FS Trigger: High on the first byte of the First Line. |
| o_byte_frame_end | 1 | Output | FE Trigger: High on the last byte of the Last Line (requires logic/counter). |
| o_byte_line_start | 1 | Output | PH Trigger: High on the first byte of Every Line. |
| o_byte_packet_done | 1 | Output | PF Trigger: High on the last byte of Every Line. |

### Supported Data Types & Serialization Ratios

The block implements a Look-Up Table (LUT) based on i_cfg_data_type to determine how many bytes to send per pixel.

| Format | Data Type | Input Group | Output Group | Packing Description |
| --- | --- | --- | --- | --- |
| RAW8 | 0x2A | 1 Pixel | 1 Byte | P1[7:0] -> Byte0 |
| YUV422 / RGB565 | 0x1E / 0x22 | 1 Pixel | 2 Bytes | P1[15:8] -> Byte0 P1[7:0] -> Byte1 |
| RGB888 | 0x24 | 1 Pixel | 3 Bytes | P1[23:16] -> Byte0 P1[15:8] -> Byte1 P1[7:0] -> Byte2 |
| RAW10 | 0x2B | 4 Pixels | 5 Bytes | Standard CSI-2 Packing: Bytes 0-3: MSBs (Bits 9:2) of Pixels 1-4. Byte 4: Aggregated LSBs (Bits 1:0) of Pixels 1-4. |

## Functional Description

1. Dynamic Serialization & Gearbox Packing

The core function is a configurable state machine that adapts its behavior based on the i_cfg_data_type. It operates in one of two modes:

- Mode A: Simple Serialization (Intra-Pixel):

Used for formats like RGB888 (1:3), RGB565 (1:2), and YUV422 (1:2).

  - Load Cycle: On valid input, the pixel is latched. Byte 0 is output immediately.

  - Busy Cycles: o_native_ready is pulled LOW. The remaining bytes (Byte 1, Byte 2, etc.) are shifted out sequentially.

  - Completion: Once the specific byte count for that format is reached, o_native_ready returns HIGH.

- Mode B: Gearbox Packing (Inter-Pixel):

Used for RAW10 (4 Pixels  5 Bytes). The block processes pixels in groups of four.

  - Accumulation (Cycles 1-4):

    - The block accepts 4 consecutive pixels.

    - For each pixel, the upper 8 bits (MSBs) are sent to o_byte_data immediately.

    - The lower 2 bits (LSBs) are stripped and stored in an internal "LSB Buffer".

  - Flush (Cycle 5):

    - After the 4th pixel, o_native_ready goes LOW (Busy) for exactly one cycle.

    - The block outputs the 5th byte, composed of the aggregated LSBs from the previous four pixels.

2. Variable Flow Control (Backpressure)

Since the serialization ratio changes based on the Data Type, the o_native_ready handshake pattern varies:

- 1:1 (RAW8): Ready stays HIGH (no backpressure).

- 1:N (RGB/YUV): Ready toggles (HIGH for 1 cycle, LOW for (N-1) cycles).

- 4:5 (RAW10): Ready stays HIGH for 4 cycles, then LOW for 1 cycle (during the LSB flush).

3. Marker Logic (Packetizer Triggers)

The converter decomposes pixel flags into byte-level triggers. The timing of these triggers is Data Type dependent.

- Start of Line (o_byte_line_start):

  - Asserts: HIGH during Byte 0 of the very first pixel of any line.

  - Usage: Triggers the Packetizer to generate the Long Packet Header (PH).

- Start of Frame (o_byte_frame_start):

  - Asserts: HIGH during Byte 0 of the first pixel, only if i_native_sof is present.

  - Usage: Triggers the Packetizer to send the Frame Start (FS) Short Packet.

- Packet Done / End of Line (o_byte_packet_done):

  - Asserts: HIGH during the Last Byte of the serialization sequence for the last pixel/group.

    - RGB888: Byte 2 of the last pixel.

    - RAW10: The 5th (LSB) byte of the last pixel group.

  - Usage: Triggers the Packetizer to calculate the Checksum and end the Long Packet.

- End of Frame (o_byte_frame_end):

  - Asserts: Coincident with o_byte_packet_done when the standard i_native_eol logic indicates the last line of the image.

## Block Diagram

## Timing Diagram

## Verification Requirements

To ensure this block functions correctly, the following scenarios must be verified:

- Protocol Compliance: Verify that o_native_ready correctly asserts and deasserts based on the selected Data Type (e.g., toggles for RGB888, pauses every 5th cycle for RAW10) to prevent upstream data loss.

- Data Integrity: Verify that incoming pixel data is correctly serialized (or packed in RAW10 mode) into the output byte stream, ensuring bit-perfect reconstruction of the image data.

- Packet Flag Alignment: Verify that o_byte_line_start asserts on the exact first byte of the line and o_byte_packet_done asserts on the exact last byte of the line (delaying the input EOL signal as needed).

- Downstream Backpressure: Perform simulation where i_byte_ready is forced Low randomly during active serialization cycles to ensure the internal FSM pauses correctly without dropping or duplicating bytes.

- Corner Case Handling: Verify that in RAW10 mode, if a line ends with fewer than 4 pixels (Partial Group), the block immediately flushes the remaining LSBs and asserts o_byte_packet_done correctly.