# MIPI Scrambler Functional Design Specifications

MIPI CSI Controller IP

Version 1.0

Block Owner

Abdelrahman Elsayed

Authors

Abdelrahman Elsayed

This Page is left Blank Intentionally

# Table of Contents

1	Table of Contents	1

2	Revision History	2

3	Overview	3

4	Operation and Description	3

4.1	Digital Interface	3

4.1.1	Parameters Names	3

4.1.2	Ports Names	3

4.2	Functional Description	3

4.3	Block Diagram	3

4.4	Timing Diagram	3

4.5	Verification Requirements	3

# Revision History

| Version | Date | Author(s) | Revision Notes | Owner Approval |
| --- | --- | --- | --- | --- |

# Overview

The Data Scrambler module is a critical transmit-side component within the MIPI CSI Controller IP. It is designed to reduce Electromagnetic Interference (EMI) by scrambling the outgoing high-speed data stream. The module utilizes a 16-bit Linear Feedback Shift Register (LFSR) with parallel feedback logic, processing one full byte per clock cycle to match the high-speed requirements of the Protocol-to-PHY Interface (PPI).

# Operation and Description

## Digital Interface

### Parameters Names

| Parameter Name | Default | Description |
| --- | --- | --- |
| LANE_SEED | 16'h0810 | The 16-bit initialization vector (seed) used to initialize the LFSR state. This can be configured per lane. |

### Ports Names

| Port Name | Port Width | Port Type | Description |
| --- | --- | --- | --- |
| i_clk | 1 | Input | System core clock. |
| i_rst_n | 1 | Input | Active-low asynchronous system reset. |
| i_scram_en | 1 | Input | Runtime enable control (1 = scramble data, 0 = passthrough bypass). |
| i_lane_data | 8 | Input | Original (plaintext) byte stream to be transmitted. |
| i_lane_vld | 1 | Input | Valid signal indicating active payload data on the input bus. |
| o_scrambled_data | 8 | Output | The scrambled (ciphered) output byte stream. |
| o_lane_vld_out | 1 | Output | The output valid signal, matching the input valid. |

## Functional Description

The operation of the MIPI Scrambler is based on a standard MIPI D-PHY scrambling polynomial and is divided into three main operational phases:

- Hardware-Unrolled Parallel Feedback Logic: To process an entire byte in a single clock cycle, the sequential LFSR equations are hardware-unrolled. Wide XOR logic computes 8 future feedback bits (w_fb[0:7]) simultaneously, allowing the LFSR to jump 8 states ahead in one cycle without stalling the high-speed pipeline.

- LFSR State Management: The LFSR state register (r_lfsr_state) initializes to LANE_SEED on hard reset. To ensure deterministic behavior and glitch-free toggling, the state is forced back to LANE_SEED whenever scrambling is disabled (i_scram_en = 0) or during inter-packet idle periods. During an active transmission, the state only advances to generate the next key when a valid payload byte is presented (i_lane_vld = 1).

- Priority MUX Output Masking: The final data output is controlled by a 3-level priority mechanism:

  - If i_lane_vld is 0, the output is clamped to 8'h00 to prevent unknown data leakage.

  - If i_scram_en is 0, the input data flows transparently to the output unaltered (Bypass mode).

  - Otherwise, the output is the bitwise XOR operation between the input data (i_lane_data) and the lower 8 bits of the LFSR (w_key_stream).

## Block Diagram

## Timing Diagram

## Verification Requirements