# Lane Distributor Functional Design Specifications

Project NameMIPI CSI Controller IP

Version 1.0

Block Owner

Abdelrahman Elsayed

Authors

Abdelrahman Elsayed

This Page is left Blank Intentionally

# Table of Contents

1	Table of Contents	1

2	Revision History	2

3	Overview	3

4	Operation and Description	3

4.1	Digital Interface	3

4.1.1	Parameters Names	3

4.1.2	Ports Names	3

4.2	Functional Description	3

4.3	Block Diagram	3

4.4	Timing Diagram	3

4.5	Verification Requirements	3

# Revision History

| Version | Date | Author(s) | Revision Notes | Owner Approval |
| --- | --- | --- | --- | --- |

# Overview

The Lane Distributor (Demultiplexer) is a critical core module within the MIPI CSI Controller IP. It is responsible for accepting a continuous, sequential byte stream from the preceding Packetizer block and dynamically distributing it across 1, 2, or 4 parallel MIPI D-PHY lanes. The distribution process adapts in real-time based on the system's active lane configuration, ensuring zero-latency data routing.

# Operation and Description

## Digital Interface

### Parameters Names

| Parameter Name | Default | Description |
| --- | --- | --- |
| N/A | N/A | This module does not utilize instantiation parameters. Lane configuration is handled dynamically via input ports. |

### Ports Names

| Port Name | Port Width | Port Type | Description |
| --- | --- | --- | --- |
| i_clk | 1 | Input | System core clock. |
| i_rst_n | 1 | Input | Active-low asynchronous system reset. |
| i_pkt_data | 8 | Input | Serial byte stream coming from the Packetizer. |
| i_pkt_vld | 1 | Input | Valid signal accompanying the incoming packet data. |
| i_cfg_active_lanes | 2 | Input | Configuration threshold defining the number of active lanes (e.g., 0 for 1-lane, 1 for 2-lanes, 3 for 4-lanes). |
| o_lane[0:3]_data | 8 | Output | Parallel data buses driving each respective MIPI lane. |
| o_lane[0:3]_vld | 1 | Output | Valid signals indicating valid parallel data is ready on the respective lane. |
| o_req_hs | 1 | Output | Look-ahead early High-Speed request signal to awaken the D-PHY. |

## Functional Description

- Dynamic Demultiplexing & Programmable Lane Counter: Seamlessly switches between 1-lane, 2-lane, and 4-lane distribution topologies. A synchronized counter (r_lane_cnt) tracks the target lane for the incoming byte. It increments on every valid byte and wraps around automatically when it hits the configured active lane threshold (i_cfg_active_lanes), ensuring cyclic distribution.

- Byte Demultiplexer: Acting as a high-speed router, it reads the current lane counter state and latches the incoming i_pkt_data into the corresponding o_laneX_data output register without introducing dead cycles.

- Valid Generation & Smart Flush Logic: During normal operation, valid signals for the parallel lanes are asserted only when a complete set of bytes is buffered. If i_pkt_vld deasserts prematurely while the counter is non-zero, the w_flush_en condition is triggered. This forces the module to safely push out partially filled parallel lane registers, preventing data loss or physical layer misalignment.

- Early HS Wake-up: To meet MIPI D-PHY timing requirements, o_req_hs is logically OR-ed with the incoming packet valid, a delayed valid, and the output lane 0 valid. This ensures the PHY transitions into High-Speed mode concurrently with the arrival of active packet data and remains active during the flush cycle.

## Block Diagram

## Timing Diagram

## Verification Requirements